Perfect da. We’ll do this in two stages exactly as you said:

Stage 1 now: understand the overall flow, function by function. No line-by-line code analysis yet.
Stage 2 later: once you say “I’m done”, we’ll dive into every important line, variable, condition, and protocol concept.

🛑 STOP-AND-WAIT ARQ: Overall Flow

You have two separate programs:

TCP Connection

┌─────────────────────────────┐

│                             │

▼                             ▼

SENDER.C                       RECEIVER.C

│                             │

│────── Frame 0 ────────────►│

│                             │

│◄──────── ACK 1 ────────────│

│                             │

│────── Frame 1 ────────────►│

│                             │

│◄──────── ACK 0 ────────────│

│                             │

│────── Frame 0 ────────────►│

│                             │

│◄──────── ACK 1 ────────────│

The central idea is extremely simple:

Sender sends ONE frame and waits for its ACK before sending the next frame.

That's why it is called Stop-and-Wait.

1. First: Program starts

Both programs have their own main().

Sender

Create socket

↓

Connect to receiver

↓

Display frame structure

↓

Display IP addresses

↓

Show menu

Receiver

Create socket

↓

Bind to port 8200

↓

Listen

↓

Accept sender connection

↓

Display frame structure

↓

Display IP addresses

↓

Wait for frames

So the receiver starts its server first.

Then sender connects to it.

2. binaryValue()

This is your new separate function.

void binaryValue(int value,char binary[])

Purpose

It takes:

Decimal number

↓

8-bit binary

For example:

127

↓

01111111

It does only the conversion.

It doesn't print anything.

3. printIPBinary()

void printIPBinary(char ip[])

This function is responsible for displaying an IP address and its binary representation.

For:

127.0.0.1

it first separates the IP into:

127

0

0

1

Then it calls:

binaryValue(127)

binaryValue(0)

binaryValue(0)

binaryValue(1)

So:

printIPBinary()

│

├── binaryValue(127)

├── binaryValue(0)

├── binaryValue(0)

└── binaryValue(1)

The important separation is:

binaryValue()     → generates binary

printIPBinary()   → prints the IP + generated binary

4. Sender creates the connection

Inside sender's main():

socket()

↓

inet_pton()

↓

connect()

The sender connects to:

127.0.0.1 : 8200

So both programs communicate through the same computer using localhost.

5. Receiver creates the server

Receiver's main() does:

socket()

↓

bind()

↓

listen()

↓

accept()

Meaning:

socket()

Creates the communication endpoint.

bind()

Associates it with:

Port = 8200

listen()

Makes the receiver wait for a connection.

accept()

Accepts the sender's connection.

After this:

Sender  <──────── TCP connection ────────>  Receiver

6. Sender displays the menu

The sender repeatedly shows:

1. Normal propagation

2. Time-out (ACK lost)

3. Frame lost

4. Error in frame

5. Exit

These are basically your four simulation cases.

Mode 1 → Normal

Mode 2 → ACK loss

Mode 3 → Frame loss

Mode 4 → Data error

7. Sender gets the message

User enters something like:

Hi

The sender calculates:

length

Then converts every character into 8-bit binary.

For example:

H → 01001000

i → 01101001

So:

Hi

↓

0100100001101001

This is stored in:

binary[]

8. Sender divides the binary into frames

Your frame contains:

16 bits of data

Therefore:

2 characters = 16 bits

So if the input is:

HELLO

you get:

HE → Frame

LL → Frame

O  → Frame

The final frame is padded with zeroes because only one character remains.

9. Sender creates a Message

This is important.

Your actual transmitted object is:

Message

Inside Message:

type

mode

totalFrames

messageLength

frame

And inside frame:

Source IP

Destination IP

Sequence number

16-bit data

Parity bit

So conceptually:

MESSAGE

│

├── type

├── mode

├── totalFrames

├── messageLength

│

└── FRAME

├── Source IP

├── Destination IP

├── Seq No

├── Data

└── Parity

10. Sequence number Sn

This is the heart of Stop-and-Wait.

Your protocol uses:

0

1

0

1

0

1

...

So:

Sn = 0

Sender sends:

Frame 0

After receiving correct ACK:

Sn = 1

Then:

Frame 1

After correct ACK:

Sn = 0

Then:

Frame 0

And so on.

This is why you see:

Frame 0 SENT

ACK 1 RECEIVED

Frame 1 SENT

ACK 0 RECEIVED

The ACK indicates the next expected sequence number.

11. Sender calculates parity

For every 16-bit data field:

0110100001100101

Sender counts how many 1s are present.

Then:

parityBit = number_of_ones % 2

So the frame becomes:

DATA              PARITY

0110100001100101    1

The parity is sent along with the frame.

12. Sender sends the frame

Eventually:

send(sockfd,&msg,sizeof(msg),0);

happens.

Conceptually:

Sender

│

│ Frame 0

│

└──────────────────────►

Receiver

Then sender waits.

This is the STOP part of Stop-and-Wait.

13. Receiver receives the frame

Receiver executes:

recv()

and obtains the Message.

Then it looks at:

msg.type

msg.mode

msg.frame

If it is a frame:

MSG_FRAME

it starts processing it.

14. Receiver checks parity

Receiver takes the received 16 data bits.

It counts the 1s again.

Then:

calculated parity

↓

compare

↓

received parity bit

If equal:

Parity Bit Verified

Frame can continue.

If different:

PARITY BIT ERROR

Frame DISCARDED

No ACK

The receiver does not accept the corrupted frame.

15. Receiver checks sequence number Rn

Receiver maintains:

Rn

This means:

The sequence number of the frame I am currently expecting.

Initially:

Rn = 0

So receiver expects:

Frame 0

If:

received seqNo == Rn

then:

Expected frame

It accepts it.

16. Receiver stores the data

Once the frame is valid:

16-bit data

↓

binary[]

The receiver stores the bits sequentially.

For example:

Frame 0 → first 16 bits

Frame 1 → next 16 bits

Frame 0 → next 16 bits

...

And:

receivedFrames++

17. Receiver changes Rn

After accepting Frame 0:

Rn = 1

After accepting Frame 1:

Rn = 0

So:

Frame 0 accepted

↓

Rn becomes 1

↓

ACK 1

Then:

Frame 1 accepted

↓

Rn becomes 0

↓

ACK 0

This is why your ACK pattern is:

Frame 0 → ACK 1

Frame 1 → ACK 0

Frame 0 → ACK 1

18. Receiver sends ACK

Receiver creates:

Ack ack;

and sets:

ack.ackNo = Rn

Then:

send(clientfd,&ack,sizeof(ack),0);

So:

Receiver

│

│ ACK 1

▼

Sender

Sender receives it.

19. Sender validates ACK

Sender expects:

(Sn + 1) % 2

For:

Sn = 0

expected ACK:

(0 + 1) % 2

= 1

For:

Sn = 1

expected ACK:

(1 + 1) % 2

= 0

So:

Frame 0 → ACK 1

Frame 1 → ACK 0

If the ACK is valid:

Stop timer

Sn changes

Next frame can be sent

20. What happens when all frames are received?

Receiver checks:

receivedFrames == totalFrames

When true:

All frames received

Then it takes the complete binary sequence:

0100100001101001...

and converts every 8 bits back into a character.

So:

01001000 → H

01101001 → i

Finally:

MESSAGE : Hi

So the complete journey is:

USER INPUT

↓

Characters

↓

8-bit binary

↓

16-bit frames

↓

Add sequence number

↓

Add parity

↓

SEND

↓

RECEIVER

↓

Check parity

↓

Check sequence number

↓

Store data

↓

Send ACK

↓

Next frame

↓

All frames complete

↓

Binary → Characters

↓

Original MESSAGE

21. The four modes

The common flow above is modified slightly depending on the selected mode.

Mode 1: Normal propagation

Frame 0

↓

ACK 1

↓

Frame 1

↓

ACK 0

↓

Frame 0

Nothing is deliberately lost or corrupted.

Mode 2: ACK lost

Frame 0

↓

Receiver receives it

↓

ACK 1 is deliberately NOT sent

↓

Sender waits

↓

5-second timeout

↓

Retransmit Frame 0

↓

Receiver detects duplicate

↓

ACK 1

↓

Sender continues

The important point:

Frame 0 is NOT changed to Frame 1 during retransmission.

Sn stays 0 until the valid ACK arrives.

Mode 3: Frame lost

The sender deliberately doesn't transmit the first frame.

Frame 0

X

LOST

↓

No ACK

↓

Timer expires

↓

Frame 0 retransmitted

↓

ACK 1

Mode 4: Error in frame

Sender changes one data bit before transmission.

Correct data

↓

One bit changed

↓

Frame sent

↓

Receiver calculates parity

↓

Mismatch

↓

Frame discarded

↓

No ACK

↓

Sender timeout

↓

Correct bit restored

↓

Frame retransmitted

Then normal communication resumes.

🧠 The entire program in one picture

SENDER

│

│

Get user input

│

▼

Convert to binary

│

▼

Make 16-bit frame

│

▼

Add Seq + Parity

│

▼

send()

│

▼

┌───────────────┐

│    NETWORK    │

└───────────────┘

│

▼

RECEIVER

│

▼

recv() frame

│

▼

Check parity

│

┌─────┴─────┐

│           │

ERROR       CORRECT

│           │

Discard          ▼

Check Rn

│

▼

Accept frame

│

▼

Store data

│

▼

Update Rn

│

▼

Send ACK

│

▼

┌─────────────┐

│    SENDER   │

└─────────────┘

│

▼

Check ACK

│

▼

Update Sn

│

▼

Next frame

That's the function-to-function and overall protocol flow without getting into the weeds of individual statements yet.

When you say “I’m done”, we'll switch gears and go through the code itself carefully, starting from binaryValue() and printIPBinary(), then the structures, then main(), and finally the four modes.